### Project Summary: RISC-V 3-Stage Pipelined Processor

Project Name: RISC_3_Stage
Toolchain: Xilinx Vivado 2024.2
Top-Level Module: RISCV_32I_TOP.v

### Overview

This project implements a 32-bit RISC-V (RV32I) processor with a 3-stage pipeline architecture. The design integrates separate instruction and data caches, control/status register (CSR) support, and simulation-driven trace logging for debugging and verification.

The pipeline stages are organized as follows:

- Stage 1: Instruction Fetch (IF)

  - Program Counter (pc.v)

  - Instruction Memory (I_cache)

  - Pipeline Register Block 1 (reg_block_1)

- Stage 2: Instruction Decode (ID)

  - Instruction Decoder (msrv32_decoder.v)

  - Immediate Generator (imm_generator.v)

  - Immediate Adder (imm_adder.v)

  - Branch Unit (branch_unit.v)

  - Integer Register File (msrv32_integer_file.v)

  - Control Unit (machine_control.v)

  - CSR File (msrv32_csr_file.v)

  - Write Enable Generator (write_enable_gen.v)

  - Pipeline Register Block 2 (reg_block_2)

- Stage 3: Execute / Memory / Writeback (EX/WB)

  - Arithmetic Logic Unit (ALU.v)

  - Load Unit (load_unit.v)

  - Store Unit (store_unit.v)

  - Writeback Mux Unit (wb_mux_sel_unit.v)

  - Data Memory (D_cache)

### Key Features

- RV32I ISA Compliance: Supports base integer instruction set.

- 3-Stage Pipeline: Efficient instruction throughput with IF, ID, and EX/WB stages.

- CSR Integration: Provides system-level control and status functionality.

- Branch and Immediate Handling: Dedicated branch unit and immediate adder.

- Modular Memory Design: Separate instruction (I_cache) and data (D_cache) memories.

- Trace and Debug Support:

  - fwrite_file1.v module logs PC values, register writes, and memory operations to result_status.txt during simulation.

  - Facilitates step-by-step analysis of processor state.

### Applications

This design serves as a platform for:

- Studying pipeline behavior in RISC-V architectures.

- Developing and testing compiler-generated RISC-V programs.

- Extending with custom instructions for application-specific accelerators (e.g., AI/ML, DSP).

- Educational and research purposes in computer architecture and VLSI design.

In my zip file I have kept Assembly instruction file ,memory file and result status file.

this file is loaded in Icache module and Dcase module